Objective: Perform a basic Threat Modeling analysis on our Automated Plant Watering system to identify potential security vulnerabilities and propose mitigations.

1. STRIDE Threat Model Analysis: We analyzed our IoT architecture using the STRIDE methodology:

| Threat Category | Potential Vulnerability in our System | Proposed Mitigation |
| --- | --- | --- |
| Spoofing | A malicious device could pretend to be our moisture sensor and send fake "dry" data, causing the pump to run endlessly. | Use Azure IoT Hub X.509 Certificates or secure SAS tokens per device. Never hardcode credentials in plain text. |
| Tampering | An attacker physically accesses the Wio Terminal/Raspberry Pi and rewires the relay to a different appliance. | Implement physical security (enclosures) and tamper-detection switches if deployed in a public farm. |
| Repudiation | The system turns on the pump, but there is no log proving why or when it was triggered. | Enable Azure Monitor and Application Insights to keep immutable logs of all Direct Method invocations and Event Hub triggers. |
| Information Disclosure | Telemetry data (e.g., farm temperature) is intercepted over the network. | Ensure all MQTT traffic to IoT Hub is encrypted using TLS 1.2 (already handled by Azure IoT SDK). |
| Denial of Service (DoS) | A compromised script sends millions of MQTT messages per minute, draining the IoT Hub quota. | Implement rate limiting on the device side and set up billing alerts in the Azure Portal to detect spikes. |

Conclusion:

By migrating to Azure IoT Hub, our system automatically benefits from TLS encryption and per-device authentication, mitigating the biggest risks of local MQTT networks (Spoofing and Information Disclosure). Future improvements should focus on physical security for the sensors deployed in the field.