# Assignment 3: Research Sensors and Actuators

1. Sensor: DHT11 (Temperature & Humidity Sensor)

What it does:

The DHT11 is a basic, ultra-low-cost digital temperature and humidity sensor. It is widely used in the field of IoT—serving smart home systems, weather stations, and environmental monitoring applications—to measure ambient air temperature and humidity.

The electronics/hardware used inside:  This device consists of two main hardware components: a capacitive humidity sensing element and a thermistor for measuring temperature. Additionally, it integrates a basic Analog-to-Digital Converter (ADC) IC, which functions to process the analog signals from the aforementioned components and convert them into digital signals.

Is it analog or digital:  It is a Digital sensor. Although it measures analog physical properties (heat and moisture), the built-in IC outputs a formatted digital signal that can be easily read by a microcontroller's digital pins.

Units and range of measurements:

Temperature:  Measured in Celsius (°C). Range: 0°C to 50°C (with ±2°C accuracy).

Humidity: Measured in Percentage of Relative Humidity (% RH). Range: 20% to 90% RH (with ±5% accuracy).

---

2. Actuator: SG90 Micro Servo Motor

What it does:

A servo motor is a rotary actuator that enables precise control of angular position. Unlike conventional motors, which rotate continuously, a servo motor can be controlled to rotate to a specific angle and maintain that position. In the field of IoT, this type of motor is used for smart locks, moving robotic arms, or adjusting smart blinds.

The electronics/hardware used inside:  Inside the plastic casing, there is a small DC motor, a set of reduction gears (to increase torque), a potentiometer (to measure the current angle of the shaft), and a small control circuit board that processes incoming signals and adjusts the motor accordingly.

Is it analog or digital:  It is controlled via Digital signals, specifically using PWM (Pulse Width Modulation). The width of the digital pulse sent to the servo determines the angle to which it will rotate.

Units and range of inputs:

Input: Controlled by PWM pulse width, typically ranging from 1ms to 2ms (milliseconds).

Output Unit:  Degrees (°).

Range: It typically has a rotation range of 0° to 180°.